COURSE NAME: Wireless Communication                                      COURSE CODE: 5CS5-11

QUESTION BANK/ASSIGNMENT 1

SESSION 2024-25

Common to CS A,CS B, CS C

1-8(part A)

9-23(part B,C)

Note:1. Mention Proper Diagram Where ever necessary.(maximum question needs the Diagram)

2. Mention proper equation,derivation.formula or relation ship where evere necessary.

Q.1 What is fading in wireless channels and its types.

Q.2 What is slow fading.

Q.3 List the names of different path loss models.

Q.4 Explain the concept of coherence bandwidth.

Q.5 Describe the four application of wireless communication.

Q.6 List the name of different fading channels used in wireless communication.

Q.7 Explain Doppler shift.

Q.8 Explain link Budget design.

Q.9 Explain Doppler spread and coherence time in detail.

Q.10 A mobile is moving at 60m/s in a cellular system with 930 Mhz.Find the Doppler spread,coherence time and appropriate sampling time and distance to predict small-scale-fading.

Q.11 if a transmitter produces 50W of power,express the transmit power in units of (a) dBm (b) dBw. If 50 W is applied to a unity gain antenna with a 900 MHz  carrier frequency, find the received power in dBm at a free space distance of 100m from the antenna. What is Pr (10 km)? Assume unity gain for the reveiver antenna.

Q.12 What is Fading?Explain the multipath propagation phenomena with the help of fading diagram.

Q.13 Describe fast fading and slow fading.

Q.14 Determine the critical distance for the two ray model in an urban microcell (ht = 10m , hr =3m) & an Indoor microcell (ht = 3m , hr =2m) for Fc =2Ghz).

Q.15 A mobile is located 5km away from a base station & uses a vertical λ/4 monopole antenna with a gain of 2.55 dB to receive cellular radio signal.

The E- Field at 1 km from the transmitter is measured to be 10-3 V/m.The carrier frequency used for this system is 900 MHz

(a) Find the length & the effective apperature of the receiving antenna.

(b) find the received power at the mobile using the two-ray ground reflection model assuming the height of the transmitting antenna is 50m & the receiving antenna is 1.5m above ground.

Q.16 Suppose the mean path loss of a particular T-R , communication link is predicted to be 138 dB. The transit power of 10W (40dBm). Through measurement , it was found that the standard deviation of the log-normal shadowing is 5dB (averaged over many similar locations at roughly the same distance).

Calculate the probability that the received signal power is more than -95dBm.Also calculate the outage probability.

Q.17 Find the outage probability at 150m for a channel based on the combined path loss & shadowing with path loss exponent n = 3.71, mean path loss, K = -31.54 dB (at close in distance d0 = 1m) & standard deviation 3.65 dB. Assume transmit power of Pt =10mW & min power requirement of Pmin = -110.5dBm.

Q.18 Find the doppler shift if the carrier frquency, Fc = 900MHz & the vehicle is moving at 60 Km/hr.

(a) for ,if vehicle is moving towards the Tx (θ=0)

(b) For,if vehicle is moving away from the Tx (θ=1800)

(c) If the vehicle is moving perpendicular to the angle of arrival of the transmitted signal  (θ=?)

Q.19 Find the Fraunhofer distance for an antenna with maximum dimension of 1m and operating frequency of 900 MHz. If antennas have unity gain,calculate the path loss.

Q.20 Describe large scale path loss.Also define path loss models in detail.

Q.21 What is Multiple Access Techniques ? Differentiate between FDMA,CDMA and TDMA.Also Explain their working.

Q.22 Explain the different parameters of mobile multipath channel.

Q.23 Explain the fading effects due to multipath time delays spread and fading effects due to doppler spread.