COURSE NAME: Principle of Electronic Communication                          COURSE CODE: 7EC6.60.1

SESSION 2025-26

What is amplitude modulation?

Ans: The process of amplitude modulation consists of varying the peak amplitude of a

sinusoidal carrier wave in proportion to the instantaneous amplitude of the modulation signal.

What are the main components needed for a communication?

Ans: Sender, Transmission line and Receiver.

What is Radio communication?

Ans: Radio communication uses electrical energy to transmit information. Radio transmitter converts audio (sound) signals to electrical signals that are sent over wires or through space. A radio receiver converts the electromagnetic waves so that the information can be found out.

Range of Audio Frequency?

Ans: 20Hz to 20 KHz

Range of Radio Frequency?

Ans: 10 KHz to 1000GHz.

What is modulation?

Ans: Modulation is the process of changing the characteristics of the carrier signal with respect to that of the message signal.

What are the principle forms of modulation?

Ans: Amplitude Modulation (AM)

Frequency Modulation (FM)

Phase Modulation (PM)

What are the types of angle modulations?

Ans: FM and PM

What is a mixer?

Ans: An electronic circuit that combines two frequencies.

What is known as Bandwidth?

Ans: Bandwidth is the frequency range, in hertz (Hz), between the upper and lower frequency limits.

What is Harmonics?

Ans: Harmonics is the signals with frequencies that are an integral multiple of the fundamental frequency.

What is the range of frequency in commercial AM broadcasting?

Ans: 540 KHz to 1600 KHz has a 10 KHz bandwidth.

Range of frequency in AM baseband?

Ans: 100KHZ to 5 KHz.

What is baseband?

Ans: It is a band of frequencies occupied by a message signal.

What you mean by frequency translate?

Ans: It is the process of displaying the frequency contents of a signal to another place in the frequency spectrum.

Modulation index?

Ans: Modulation index (m) is the ratio between the amplitude of the message signal to the amplitude of the un modulated carrier signal.

What is percentage of modulation?

Ans: The modulation index (M) is expressed as a percentage.(%m)

Transmission efficiency?

Ans: It is the ratio of the total AM power to that of the sideband.

What is envelope detector?

Ans: A circuit containing a diode in series with an RC network, used to perform demodulation.

An envelope detector, which demodulates an AM signal

What is centre frequency?

Ans: Centre frequency is the frequency of the un modulated FM carrier signal.

What is frequency deviation?

Ans: Frequency deviation – the maximum frequency change between a modulated and un modulated carrier signal.

What is discriminator?

Ans: Discriminator is a device that demodulates an FM signal.

lOMoARcPSD|28264989

What is Quadrature?

Ans: Quadrature is the phase relationship between two equal frequency signals that are 90° out of phase.

FM modulation index?

Ans: FM modulation index – the ratio of frequency deviation to the message signal frequency.

What is signal to noise ratio?

Ans: A signal-to-noise ratio is the ratio of the magnitude of the signal to that of noise (often expressed in decibels).

What is Frequency-Shift Keying (FSK)?

Ans: Frequency-Shift Keying (FSK) is shifting an analog signal’s frequency between two values to represent the binary states high and low of a digital signal. A modem uses FSK to transmit digital signals over analog telephone lines.

What is noise?

Ans: An unwanted signal that will disturb the transmission or processing of signals in

communication systems. Eg. Shot noise, Thermal noise.

What is modulation?

Ans: A process by which some characteristic of a carrier is varied in accordance with a

modulating wave (baseband signal).There are, Amplitude modulation

Angle modulation (Phase modulation, Frequency modulation).

What is Transmission Bandwidth?

Ans: Transmission bandwidth of an AM wave,

For positive frequencies, the highest frequency component of the AM wave

equals fc + W, and the lowest frequency component equals fc – W.

The difference between these two frequencies defines the transmission

bandwidth BT for an AM wave.

Limitations of Amplitude Modulation?

Ans: Waste of power in the information-less “with-carrier” part.

Wasteful of power and bandwidth

What is Angle Modulation?

Ans: The angle of the carrier is varied in accordance with the base band signal.

Commonly used angle modulation

Phase modulation (PM)

Frequency modulation (FM)

What is frequency translation?

Suppose that a signal is band limited to the frequency range extending from a frequency

f1 to a frequency f2. The process of frequency translation is one in which the original signal is replaced with a new signal whose spectral range extends from f1‘ and f2‘ and which new signal

bears, in recoverable form the same information as was borne by the original signal.

Define depth of modulation.

It is defined as the ratio between message amplitude to that of carrier amplitude.

m=Em/Ec

What are the degrees of modulation?

Under modulation. m<1

Critical modulation m=1

Over modulation m>1

What is the need for modulation?

Needs for modulation:

Ease of transmission

Multiplexing

Reduced noise

Narrow bandwidth

Frequency assignment

Reduce the equipments limitations

What are the types of AM modulators?

There are two types of AM modulators. They are

Linear modulators

Non-linear modulators

Linear modulators are classified as follows

Transistor modulator

There are three types of transistor modulator.

Collector modulator

Emitter modulator

Base modulator

Switching modulators

Non-linear modulators are classified as follows

Square law modulator

Product modulator

Balanced modulator

What is the difference between high level and low level modulation?

In high level modulation, the modulator amplifier operates at high power levels and

delivers power directly to the antenna. In low level modulation, the modulator amplifier

performs modulation at relatively low power levels. The modulated signal is then amplified to high power level by class B power amplifier. The amplifier feeds power to antenna.

Define Detection (or) Demodulation.

Detection is the process of extracting modulating signal from the modulated carrier.

Different types of detectors are used for different types of modulations.

Define Amplitude Modulation.

In amplitude modulation, the amplitude of a carrier signal is varied according to

variations in amplitude of modulating signal.

The AM signal can be represented mathematically as, eAM = (Ec + Em sinωmt) sinωct and the modulation index is given as,m = Em /EC (or) Vm/Vc

Define Demodulation.

Demodulation or detection is the process by which modulating voltage is recovered from

the modulated signal. It is the reverse process of modulation. The devices used for

demodulation or detection are called demodulators or detectors. For amplitude modulation, detectors or demodulators are categorized as,

a) Square-law detectors

b) Envelope detectors

Define Multiplexing.

Multiplexing is defined as the process of transmitting several message signals

simultaneously over a single channel.

Define Frequency Division Multiplexing.

Frequency division multiplexing is defined as many signals are transmitted

simultaneously with each signal occupying a different frequency slot within a common

bandwidth.

What is BW for AM wave?

The difference between these two extreme frequencies is equal to the bandwidth of the AM wave.

Therefore, Bandwidth, B = (fc + fm) - (fc - fm) B = 2fm

What do you understand by narrowband FM?

When the modulation index is less than 1, the angle modulated systems are called low index.

The bandwidth requirement of low index systems is approximately twice of the modulating.

Define frequency modulation.

Frequency modulation is defined as the process by which the frequency of the carrier wave is varied in accordance with the instantaneous amplitude of the modulating or message signal.

Define modulation index of frequency modulation.

It is defined as the ratio of maximum frequency deviation to the modulating

What do you meant by multi tone modulation?

Modulation done for the message signal with more than one frequency component is called multi tone modulation.

Define phase modulation.

Phase modulation is defined as the process of changing the phase of the carrier signal in  accordance with the instantaneous amplitude of the message signal.

What are the types of Frequency Modulation?

Based on the modulation index FM can be divided into types. They are Narrow band FM and Wide band FM. If the modulation index is greater than one then it is wide band FM and if the modulation index is less than one then it is Narrow band FM

What is the basic difference between an AM signal and a narrowband FM signal?

In the case of sinusoidal modulation, the basic difference between an AM signal and a

narrowband FM signal is that the algebraic sign of the lower side frequency in the narrow band FM is reversed.

What are the two methods of producing an FM wave?

Basically there are two methods of producing an FM wave. They are,

i) Direct method: In this method the transmitter originates a wave whose frequency varies as function of the modulating source. It is used for the generation of NBFM

ii) Indirect method: In this method the transmitter originates a wave whose phase is a

function of the modulation. Normally it is used for the generation of WBFM where WBFM is generated from NBFM.

Compare WBFM and NBFM.

Give the average power of an FM signal.

The amplitude of the frequency modulated signal is constant .The power of the FM signal is same as that of the carrier power.

Define phase deviation.

The maximum phase deviation of the total angle from the carrier angle is called phase

deviation.

Define frequency Deviation.

The maximum departure of the instantaneous frequency from the carrier frequency is called  frequency deviation.

State the Carson‘s rule.

An approximate rule for the transmission bandwidth of an FM Signal generat ed by a single tone-modulating signal of frequency f m (max) is defined as

∴ BW=2[δ + fm(max)]

What are the disadvantages of FM system?

1. A much wider channel is required by FM.

2. FM transmitting and receiving equipments tend to be more complex and hence it is

expensive.

What are the types of FM detectors?

The types of FM detectors are

(i) Slope detector and (ii) Phase discriminator.

Differentiate phase and frequency modulation.

Phase Modulation:

Phase of the carrier varies as per amplitude variations of modulating signal.

Instantaneous phase deviation, (t ) = k em (t )

Modulation index = k Em

Frequency Modulation:

Frequency of the carrier varies as per amplitude variations of modulating signals.

Instantaneous frequency deviation, Δ (t ) = k em (t )

Modulation index = k Em

What are the advantages of Analog Communications?

Transmitters and Receivers are simple

Low bandwidth requirement

FDM can be used

What are the disadvantages of analog communication?

Noise affects the signal quality

It is not possible to separate noise and signal

Repeaters can‘t be used between transmitters and receivers

Coding is not possible

It is not suitable for the transmission of secret information

UNIT-2

What are the types of Digital Modulation Techniques? Explain.

Ans. basically, digital modulation techniques may be classified into coherent or non-coherent techniques, depending on whether the receiver is equipped with a phase-recovery circuit or not.

The phase-recovery circuit ensures that the oscillator supplying the locally generated carrier wave to the receiver is synchronized to the oscillator supplying the carrier wave used to originally modulate the incoming data stream in the transmitter.

(i) Coherent Digital Modulation Techniques

Coherent digital modulation techniques are those techniques which employ coherent detection.

In coherent detection, the local carrier generated at the receiver is phase locked with the carrier at the transmitter. Thus, the detection is done by correlating received noisy signal and locally generated carrier. The coherent detection is a synchronous detection.

(ii) Non-coherent Digital Modulation Techniques

Non-coherent Digital Modulation Techniques are those techniques in which the detection process does not need receiver carrier to be phase locked with transmitter carrier.

The advantage of such type of system is that the system becomes simple. But the drawback of such system is that the error probability increases.

State various continuous wave (CW) modulation systems.

Ans. ASK, FSK, PSK, QPSK, MSK, M-ary PSK etc.

Represent ASK mathematically.

Ans. VASK(t) = d(t) sin (2πfct) where d(t) = 1 or 0 .

State the bandwidth requirement of ASK system.

Ans. Maximum bandwidth Bmax = 2 fb Hz.

Why is PSK always preferable over ASK in coherent detection?

Ans. ASK is on-off signaling where as the modulated carrier is continuously transmitted in PSK. Hence peak power requirement is more in ASK, where it is reduces in PSK.

Differentiate between coherent and non-coherent detection

Ans. In coherent detection the local carrier generated at the receiver is phase locked with the carrier at the transmitter. Hence it is also called synchronous detection. In non coherent detection the local carrier generated at the receiver not be phase locked with the carrier at the transmitter. It is simple, but it has higher probability of error.

Define QPSK.

Ans. In QPSK two successive bits in the data sequence are grouped together. This

combination of two bits forms four distinct symbols. When the symbol is changed to next

symbol the phase of the carrier is changed by 450(or π/4).

Because of combination of two bits there will be four symbols. Hence the phase shift will

be π/4, 3π/4, 5π/4 or 7π/4.

QPSK reduces amplitude variations and required transmission bandwidth.

Define Amplitude Shift Keying (ASK).

Amplitude Shift Keying is defined as changing the amplitude of the carrier signal with respect to the binary information or digital signal.

Define Phase Shift Keying (PSK).

Phase Shift Keying is defined as changing the amplitude of the carrier signal with respect to the binary information or digital signal.

Define Frequency Shift Keying (FSK).

Frequency Shift Keying is defined as changing the amplitude of the carrier signal with respect to the binary information or digital signal.

What are the advantages of QPSK?

Very good noise immunity

Effective utilization of available bandwidth

Low error probability

Very high bit rate data transmission

UNIT-3

What is VoIP?

Ans: VoIP stands for Voice over Internet Protocol or Voice over IP. VoIP technology makes it possible to convert analog voice signal into digital data and transmits it over the Internet.(There are more likely possible pronunciations, as well as vo-ipp, have been used, but generally, the single syllable - voyp, as in voice - may be the most common within the industry.)

Why VoIP is better than traditional phone services?

Ans: Due to its cost efficiency, VoIP is more and more popular largely over traditional

telephone networks. VoIP cuts companies’ monthly phone bill by approximately fifty percent.

In addition to its cost efficiency, VoIP technology ensures many advanced features, like

conference calling, IVR, call forwarding, automatic redial, call recording, etc. without extra fees.

VoIP offers cheaper international long distance rates that are generally one-tenth of what is charged by traditional phone companies. Due to its portability VoIP is a really good option to avoid expensive hotel phone and cell phone roaming charges.

Only a high speed broadband connection (and a plugged adapter) is needed and anyone can reach you at your local number - independently of your location. Most of the times in network calls to other VoIP service subscribers are free even if the calling parties are located in different parts of the world.

By using Internet connection for both data traffic and voice calls, it is possible to get rid of one monthly payment that are usually charged by most Internet service providers. In addition, the Internet based voice and data transmission enables to avoid wireless roaming fees and long distance rates.

What are the advantages of VoIP?

Ans: In addition to its cost efficiency, the feature-rich services and the metaphorical

disappearance of geographical boundaries as that were mentioned above; VoIP has many other benefits as follows:

VoIP technology enables to detect and process touch tones and DTMF responses

VoIP systems can be automated easily

VoIP systems allow to use more than one codec

VoIP provides rich media service as more file formats can be used with these systems

VoIP ensures a much more flexible system than hardware based solutions

Most VoIP service providers provide a user control interface, typically a web GUI, to their customers

so that they can change features, options, and services dynamically. VoIP protocols run on the application layer and are able to integrate or collaborate with other

Applications such as email, web browser, instant messenger, social-networking applications, etc.

How does VoIP work?

Ans: VoIP is usually based on the SIP system that is the recognized standard. Any SIP

compatible device can talk to any other. Any SIP telephone can call another over the Internet - any additional equipment is not needed for this. You only need to plug your SIP phone into the Internet connection, configure it then dial the other person. You can also connect traditional analog phones to your VoIP network – in this case and ATA device is needed.

In VoIP systems, your analog voice is converted into packets of data (as little files), and then transmitted to the recipient through the Internet and decoded back into your voice at the other end. To make it quicker, these packets are compressed before transmission, a bit like zipping a file (it will be decompressed of course at the other party).

The advantages of converting analog signals into digital data can be summarized as follows:

Digital format can be better controlled as it can be compressed, routed, converted, etc. In addition, digital signals are more noise tolerant than analog signals. Quality of Service (QoS) ensures real-time errorless data streaming that allows interactive data voice exchange as well.

Is it possible to replace the current traditional corporate PBX with a VoIP one?

Ans: Definitely yes. VoIP is a very cost-effective option for those companies who want to upgrade their old PBX systems and VoIP ensures new features that traditional PBX systems simply do not. To change to a VoIP system, companies can buy an IP PBX, but it is also possible to add some VoIP functionalities into an existing phone system.

What kind of equipments do I need for creating a VoIP system?

Ans: Getting started with VoIP is fairly simple. Assuming that you already have the 2 most important ingredients (a Windows PC or Mac computer and a broadband Internet connection), all you need to get started is the following: Some telephone or messaging software A microphone Headphones or speakers You can use a headset of course rather than a microphone and speaker to leave your hands free.

In order to choose which software to use, it is worth to consider the followings. Using

voice chat in GTalk or Yahoo Messenger could be regarded as VoIP, so could the highly publicized Skype; but these are all proprietary systems. You can download them free of charge, but to talk to someone using GTalk, the person at the other end also needs G-Talk. The same applies to Yahoo and, to a great extent, to Skype. They use their own special system that is not open and will not connect to other systems easily. So – especially for corporate using – it is rather recommended to use such a softphone as Conter Path X-Lite with a SIP enabled IP PBX or access to an Internet Service Provider.

What are the three criteria necessary for an effective and efficient network?

The most important criteria are performance, reliability and security.

Performance of the network depends on number of users, type of transmission medium,

and the capabilities of the connected h/w and the efficiency of the s/w.

Reliability is measured by frequency of failure, the time it takes a link to recover from

the failure and the network’s robustness in a catastrophe.

Security issues include protecting data from unauthorized access and viruses.

Group the OSI layers by function?

The seven layers of the OSI model belonging to three subgroups.

Physical, data link and network layers are the network support layers; they deal with the

physical aspects of moving data from one device to another.

Session, presentation and application layers are the user support layers; they allow

interoperability among unrelated software systems.

The transport layer ensures end-to-end reliable data transmission.

What are header and trailers and how do they get added and removed?

Each layer in the sending machine adds its own information to the message it receives

from the layer just above it and passes the whole package to the layer just below it. This

information is added in the form of headers or trailers. Headers are added to the message at the layers 6,5,4,3, and 2. A trailer is added at layer2. At the receiving machine, the headers or trailers attached to the data unit at the corresponding sending layers are removed, and actions appropriate to that layer are taken.

What are the features provided by layering?

Two nice features:

· It decomposes the problem of building a network into more manageable components.

· It provides a more modular design.

Why are protocols needed?

In networks, communication occurs between the entities in different systems. Two

entities cannot just send bit streams to each other and expect to be understood. For

communication, the entities must agree on a protocol. A protocol is a set of rules that govern data communication.

What are the two interfaces provided by protocols?

· Service interface

· Peer interface

Service interface- defines the operations that local objects can perform on the protocol.

Peer interface- defines the form and meaning of messages exchanged between protocol peers to implement the communication service.

Mention the different physical media?

· Twisted pair (the wire that your phone connects to)

· Coaxial cable (the wire that your TV connects to)

· Optical fiber (the medium most commonly used for high-bandwidth, long-distance links)

· Space (the stuff that radio waves, microwaves and infra red beams propagate through)

What are the responsibilities of data link layer?

Specific responsibilities of data link layer include the following. a) Framing b) Physical

addressing c) Flow control d) Error control e) Access control.

What are the ways to address the framing problem?

· Byte-Oriented Protocols (PPP)

· Bit-Oriented Protocols (HDLC)

· Clock-Based Framing (SONET)

What are the functions of MAC?

MAC sub layer resolves the contention for the shared media. It contains synchronization, flag, flow and error control specifications necessary to move information from one place to another, as well as the physical address of the next station to receive and route a packet.

What are the functions of LLC?

The IEEE project 802 models take the structure of an HDLC frame and divides it into 2 sets of functions. One set contains the end user portion of the HDLC frame – the logical address, control information, and data. These functions are handled by the IEEE 802.2 logical link control (LLC) protocol.

What is Ethernet?

Ethernet is a multiple-access network, meaning that a set of nodes send and receive frames over a shared link.

Define the term carrier sense in CSMA/CD?

All the nodes can distinguish between idle and a busy-link and “collision detect” means that a node listens as it transmits and can therefore detect when a frame it is transmitting has interfered (collided) with a frame transmitted by another node.

Define Repeater?

A repeater is a device that forwards digital signals, much like an amplifier forwards analog signals. However, no more than four repeaters may be positioned between any pairs of hosts, meaning that an Ethernet has a total reach of only 2,500m.

Define collision detection?

In Ethernet, all these hosts are competing for access to the same link, and as a consequence, they are said to be in the same collision detection.

Why Ethernet is said to be an I-persistent protocol?

An adaptor with a frame to send transmits with probability ‘1 ‘whenever a busy line goes idle.

What are the functions of MAC?

MAC sub layer resolves the contention for the shared media. It contains synchronization, flag, flow and error control specifications necessary to move information from one place to another, as well as the physical address of the next station to receive and route a packet.

What is Ethernet?

Ethernet is a multiple-access network, meaning that a set of nodes send and receive frames over a shared link.

Define the term carrier sense in CSMA/CD?

All the nodes can distinguish between idle and a busy-link and “collision detect” means that a node listens as it transmits and can therefore detect when a frame it is transmitting has interfered (collided) with a frame transmitted by another node.

Define Repeater?

A repeater is a device that forwards digital signals, much like an amplifier forwards analog signals. However, no more than four repeaters may be positioned between any pairs of hosts,meaning that an Ethernet has a total reach of only 2,500m.

Define collision detection?

In Ethernet, all these hosts are competing for access to the same link, and as a consequence,they are said to be in the same collision detection.

Why Ethernet is said to be an I-persistent protocol?

An adaptor with a frame to send transmits with probability ‘1 ‘whenever a busy line goes idle.